SOLS –EQE ideas

# Analytical solution with the simplest step-function approximation:

We consider a solar cell with a concrete EQE (External Quantum Efficiency, i.e. the percentual ability of the cell to convert a photon into an electron). Therefore, this EQE is a function of the wavelength (λ) because the cell has different ability of photon conversion to electricity depending on the energy of the photon. For example, if the energy of the photon is lower than the energy band gap of the active layer (AL) material, the EQE will be as a first approximation 0 as no photon is absorbed at this energy range.

If the EQE is known, one can calculate the photocurrent (Jsc) of a cell when illuminated with a specific light irradiance (which typically is the AM 1.5G solar spectrum) considering no recombination losses:

Jsc= 0∞EQEλ· ϕphλ· Qe- · dλ

Where  ϕphλ is the light irradiance expressed in #photons·s-1·m-2·nm-1, and Qe- is the electric charge of an electron in absolute value. Typically, the AM1.5G solar irradiance  ϕAM1.5Gλ is expressed in W·m-2·nm-1, and therefore, the relation between both functions is:

ϕphλ= ϕAM1.5GλEph= ϕAM1.5Gλλ1240 nm/eV·Qe-

Thus, the complete Jsc expression is:

Jsc= 0∞EQEλ· ϕAM1.5Gλ· 1240λ· dλ

In the Rainbow setup measurements, the incident light is a spectral fraction of the AM1.5G solar spectrum. As a first and rude approximation we can express this incident light as a step function with a cutting wavelength (λcw):

ϕrainbowλ, λcw=  ϕAM1.5Gλ      if λ≤ λcw0                         if λ≥ λcw

Now we can split the integral in two parts, and because the second part corresponding to the defined integral from λcw to infinite is zero, the expression for Jsc is:

Jsc(λcw)= 0λcwEQEλ· ϕAM1.5Gλ· 1240λ· dλ

The derivative of  Jsc(λcw) with respect to λcw is the slope of the measured curves in the rainbow setup:

dJsc(λcw)dλcw= d0λcwEQEλ· ϕAM1.5Gλ· 1240λ· dλdλcw

If we discretise the expression, we obtain:

ΔJscΔλcw=EQE(λcw)· ϕAM1.5Gλcw· 1240λcw

Considering that Δλcw≪λcw. Finally, the equation can be rewritten as:

EQEλcw= ΔJscΔλcw· λcw ϕAM1.5Gλcw· 1240

# Analytical solution with approximated Fermi-Dirac distribution

# Numerical solution with equipment-dependent matrix

Considering the same solar cell scenario in where the Jsc could be calculated as:

Jsc= 0∞EQEλ· ϕAM1.5Gλ· 1240λ· dλ

We assume now a more realistic scenario regarding  ϕphλ in where the step function has a certain width and therefore, solving it analytically is more complicated. Thus, one has to think in a numerical solution. In the experiment, we make “n” measures of the Jsc at different cutting wavelengths (λcw) ideally evenly distributed in the wavelength dimension (∆λ) between a low wavelength (λcw,  1) and a high wavelength (λcw,  n). We can express the ith Jsc measure as a discrete sum instead of a continuous integral:

Jscλcw,  i= j=1nEQEλj· ϕλcw,  iλj· 1240λj· ∆λ

Where  ϕλcw,  i is the solar spectrum that the cell receives at λcw,  i cutting wavelength. Which is a linear equation with n summands. We can now express in the same way all the Jsc measured, in a way that we finally will have system with n lineal equations:

Jscλcw,  1= j=1nEQEλj· ϕλcw,  1 λj· 1240λj· ∆λJscλcw,  2= j=1nEQEλj· ϕλcw,  2 λj· 1240λj· ∆λ⋮⋮Jscλcw,  n-1= j=1nEQEλj· ϕλcw,  n-1 λj· 1240λj· ∆λJscλcw,  n= j=1nEQEλj· ϕλcw,nλj· 1240λj· ∆λ

Therefore, one can express this linear equations system in the matrix form, to easily solve it with numpy.linalg.lstsq function in python.

Jscλcw,  1⋮Jscλcw,  n= ϕλcw,  1 λ1· 1240λ1· ∆λ⋯ ϕλcw,  1 λn· 1240λn· ∆λ⋮⋱⋮ ϕλcw,  n λ1· 1240λ1· ∆λ⋯ ϕλcw,  n λn· 1240λn· ∆λEQEλ1⋮EQEλn

In other words:

Jsc=C· EQE

In where Jsc is a vector that contains the measured short-circuit currents at the different cutting wavelengths, C is the Casademont constant matrix, which contains all the necessary information of the spectra to numerically solve the EQE vector. Notice that C matrix depends on the complete solar spectrum ( ϕ λ) but it also depends on the measuring conditions: λcw,  1, λcw,  n, ∆λ, red/blue sweap. Therefore, one has to calibrate the equipment with a different C for different measuring conditions, but once it is done there is no need for more calibration!

The calibration could be done by (1) measuring the spectra at each cutting wavelength and directly use this measured spectra for each cutting wavelength or (2) expressing the spectra as the convolution of the full spectra and a step function, which parameters (height, FWHM, etc…) can be fitted as a function of the cutting wavelength. This last method could be better in order to reduce the number of measures for calibration.

Here, I plot the result of this method with the Jsc results of the simulations. Notice that both Real EQE (the EQE used for the simulations) and the EQE calculated with the numerical method are exactly the same (the blue line is behind the orange one).

More simulation examples of the numerical method:

In this case, I have taken 41 real spectras of the SOLS corresponding to 41 different cutting wavelength (from 350 nm to 750 nm with 10 nm steps). From those spectras, taking the measured EQE (real EQE in the plot), I have calculated its corresponding Jsc as the sumatorial formula derived before (with a resolution in wavelength of 1 nm). Now, taking this calculated Jsc as the “measured” Jsc with the corresponding spectras, I have applied the numerical solution method (the direct resolution of the matrix formula with np.linalg.lstsq function). The result is ploted here, first with 1nm resolution (each spectra in the C matrix has 1nm resolution) and afterwards with 10 nm resolution (each spectra in the C matrix has 10nm resolution). Finally the same calculations but instead of taking 41 spectra, taking 100 spectra corresponding to a full calibration (one spectra each 0.5 mm step of blue motor). The EQE is calculated with 10 nm resolution in wavelength.

Nevertheless, this method does not work with real data (as far as I have tried) and there could be different reasons:

- First there is always a “correction factor problem” (the calculated EQE is usually a factor of 4-5 lower than the measured). This could be due to the fact that in full illumination the recombination losses are generally higher and therefore the EQE at a specific wavelength is different depending on the illumination of the cell. This means that during a red-sweep opening the Jsc is related with EQE at different illuminations, which makes this calculations a lot more complex.

If that case is the most important error source, it means that only the cells with less light power EQE dependence (i.e. less recombination loses) are good for measuring the EQE.

- Errors on the calibration of the flame. Although I have calibrated the flame with my best effort, there is still a little bit of error on the calibration, which could lead to not perfectly weighted input power and wrong EQE.